Cixi Tianhao Electric Technic Co.,ltd

NO1.Sijie Village GuanFu Road Fuhai Town, Cixi, ZheJiang, China

TEL: 0086-574-23861582 FAX: 0086-574-63688108    E-mail:trade@dispensing-th.com   Http://www.dispensing-th.com

T&H Two component Mixer/Meter dispensing machine (automatic clearing)

TH-2004AB2

The TH-2004AB2 machine has automatic cleaning function and have wheel can rolling with the frame.

1.When the machine stop work, all have a little glue leave in the tube and valve, if no cleaning in time, the glue will cure in the valve and glue ,so touch the next time to using the machine, you no need to cleaning by hand, you only switch cleaning supplier systems. The cleaning liquid automatic into the valve and fitting, automatic finish cleaning. so that good your save more time to cleaning ,same good for the machine oneself maintenance.

2.The machine have wheel, so that well you to moving the machine. Very easy and simple

Features
1.TH-2004AB2.funaction is automatic mix/meter. Then handle dispensing,

2.Have a big capability cartridge, No need to transfer material, Avoids waste.

3.Adopted with the TH-700J pneumatic double-liquid valve can interrupt the flow quickly without dripping.

3. Static mixer is used which can mix liquid evenly. Mix evenly and quickly.

4.The auto-cycling function can reduce labor intensity and increase the production.

5.Digital circuit timing controlling technology was used, and the dispensing time can be set through the digital dial.

6. The pedal /ring/inductive switches can be chosen.

7. Complete with an accessory dispensing kit.

Specification
   Standards:                CE approved

Control:              by air and electric
Mixing method:        Automatic  and static mixer

Tank:                       1gallon (Other capacity can be chosen)

Valve:               TH-700J

Operating Model:      Manual, Timer/cycles                                                                                 Leak-proof function:   valve with Vacuum device

Liquid ratio:                  1:1-3:1 (can be adjusted)

Dispensing time:           0.001S-9.999S
    Cycles (interval) time:   0.1S-9.9S
     Repeatability:               +/-0.01%
    Frequency :           > 300times/minute
    Voltage:                       110V/220V 50Hz

Internal Voltage:           24 VDC
    Power:                           < 15W

Weight:                         58 kgs

Air Input:             7KgF/cm2  Dry air without lubrication

Air Output:                   0.1-7KgF/cm2

Size:                         75X42X30cm

Complete with an accessory dispensing kit

| NO. | Descriptions | QTY | Note |
| --- | --- | --- | --- |
| 1 | TH2004AB Dispenser controller | 1pcs | IN Middle box |
| 2 | 4kgs Tank | 3pcs | IN big box |
| 3 | TH-700J Valve | 1pcs | IN small box |
| 4 | Foot pedal | 1pcs | IN small box |
| 5 | Tee | 3pcs | IN Middle box |
| 6 | STATIC MIXER | 3pcs | IN Middle box |
| 7 | Electric line(plug) | 1pcs | IN small box |
| 8 | Air tube connect controller and valve | 1pcs | IN small box |
| 9 | Air tube using for controller | 1pcs | In small box |
| 10 | Glue tube for tank(short) | 3pcs | In tank |
| 11 | air tube (white) | 3pcs | In tank |
| 12 | Air pressure gauge | 1pcs | In tank |
| 13 | Handle( black) | 9pcs | In tank |
| 14 | Machine tool with adjusting ratio | 1pcs | In small box |
| 15 | Dispensing tips | 10pcs | In middle box |
| 16 | frame | 1pcs |  |
| 17 | manual book | 1pcs | in middle box |